Student Name:			Weight:	________%

Student ID:			Marks: 	__________

Lab 3: Advanced Routing Protocols

TASK 1: CONFIGURING DNS

Please attach screenshots for each step.                                                           (5 points)

The bottom left-hand corner of the screen displays the icons that represent device categories or groups, such as Routers, Switches, or End Devices. Moving the cursor over the device categories will display each individual device category. To select a device, first select the device category and then select the device that is required.

a. Select End Devices from the options in the bottom left-hand corner. Drag and drop one Generic PCs onto your design area.

b. Select Switches from the options in the bottom left-hand corner. Add a 2950-24 switch to the design area.

c. Select Connections from the bottom left-hand corner and choose Copper Straight-through to connect the devices as per below.

1. PC0 connects to Fast Ethernet 0/1 on Switch0

2. Switch0 connects to Fast Ethernet 0/2 on Server1

d. Select End Devices from the options in the bottom left-hand corner. Drag and drop one Server onto your design area.

## Step 2: Configure network devices

a. Select PC0 and choose the Desktop tab.

b. Select IP Configuration.

c. Set the IP Address, Subnet Mask, and Default Gateway using information from the table shown below.

d. Repeat steps a – c for Server1.

| Device | IP Address | Subnet Mask | Default Gateway | DNS Server |
| --- | --- | --- | --- | --- |
| PC0 | 192.168.1.10 | 255.255.255.0 | Leave Blank | 192.168.1.1 |
| Server0 | 192.168.1.1 | 255.255.255.0 | Leave Blank | 192.168.1.1 |

## Step 3: Configure DNS on Server1

- Select Server0 and choose the Services tab.

- Select DNS under services on the left-hand column.

- For DNS Service select On

- For Resource Records type www.mypage.com in the Name box. Leave the Type as A Record.

- In the Address bar add 192.168.1.1

- Select ADD – this will add www.mypage.com as a static DNS at position 0.

TASK 2: RIP   (5 points)

Please attach screenshots for each step(one per each process).

Build the following topology:

PC CONFIGURATIONS:

| S.NO | Device | IPv4 | Subnet mask | Default Gateway |
| --- | --- | --- | --- | --- |
| 1 | PC0 | 192.168.10.2 | 24 | 192.168.10.1 |
| 2 | PC1 | 192.168.10.3 | 24 | 192.168.10.1 |
| 3 | PC2 | 192.168.20.2 | 24 | 192.168.20.1 |
| 4 | PC3 | 192.168.20.3 | 24 | 192.168.20.1 |
| 5 | PC4 | 192.168.30.2 | 24 | 192.168.30.1 |
| 6 | PC5 | 192.168.30.3 | 24 | 192.168.30.1 |

ROUTERS CONFIGURATION:

| S.NO | Device | Interface | IPv4 address | Subnet mask |
| --- | --- | --- | --- | --- |
| 1 | Router0 | Gig0/0 | 192.168.10.1 | 24 |
|  |  | Se0/1/0 | 10.0.0.1 | 8 |
| 2 | Router1 | Gig0/0 | 192.168.20.1 | 24 |
|  |  | Se0/1/0 | 10.0.0.2 | 8 |
|  |  | Se0/1/1 | 11.0.0.1 | 8 |
| 3 | Router2 | Gig0/0 | 192.168.30.1 | 24 |
|  |  | Se0/1/0 | 11.0.0.2 | 8 |

PINGING BTW DIFFERENT NETWORKS:

Answer the following: {Attach screenshots)

- Ping from PC0 to PC1, PC2, PC3, PC4 & PC5.

- Are you able to ping ?

- Why or if not why so ?

BUILDING ROUTES BETWEEN NETWORKS:

- Kindly configure rip routes in Router0, Router1 & Router2. Attach screenshot for each.

- Attach the screenshots of the routes with the command : sh ip route.

PING TEST BTW DIFFERENT NETWORKS:

- Ping from PC0 to PC1, PC2, PC3, PC4 & PC5.

- Attach the screenshot:

TASK 3: OSPF  5 points

Create the following topology :

PC CONFIGURATIONS:

| S.NO | Device | IPv4 | Subnet mask | Default Gateway |
| --- | --- | --- | --- | --- |
| 1 | PC0 | 192.168.1.2 | 24 | 192.168.1.1 |
| 2 | PC1 | 192.168.2.2 | 24 | 192.168.2.1 |
| 3 | PC2 | 192.168.3.2 | 24 | 192.168.3.1 |

ROUTERS CONFIGURATIONS:

| S.NO | Device | Interface | IPv4 address | Subnet mask |
| --- | --- | --- | --- | --- |
| 1 | Router0 | Gig0/0 | 192.168.1.1 | 24 |
|  |  | Se0/1/0 | 10.0.0.1 | 8 |
| 2 | Router1 | Gig0/0 | 192.168.2.1 | 24 |
|  |  | Se0/1/0 | 10.0.0.2 | 8 |
|  |  | Se0/1/1 | 20.0.0.1 | 8 |
| 3 | Router2 | Gig0/0 | 192.168.3.1 | 24 |
|  |  | Se0/1/0 | 20.0.0.2 | 8 |

- Enable OSPF Routing on Cisco Routers. Attach screenshots for each router.

- Show ip route for each router.

- For each router, execute the following commands and attach screenshot:

  - show ip ospf database

  - show ip ospf interface

  - show ip ospf database router

TASK 4: BGP (5 points)

Create this topology:

Paste the screenshots of the following:

- What is an AS ?

- Configure BGP on both the router and attach screenshots.